Findings from MQTT Broker and Security implications observed

MQTT is a lightweight communication protocol commonly used by IoT devices to send data through a broker. As part of this task, I connected MQTT Explorer to the public broker broker.hivemq.com and tried subscribing to different MQTT topics. The # wildcard did not show the expected overall data, so I observed specific wildcards such as room/#, home/#, sensor/#, device/#, test/#, temperature/#, status/# and data/#.

Observations by topic

| Topic | Data observed | Frequency | Real/test | Sensitive data |
| --- | --- | --- | --- | --- |
| room/# | Room/session IDs and related room or device state. | Does not change frequently | Mixed | Room/session IDs; device/application information. |
| home/# | Temperature, humidity, lights, fans, AC, plugs, doors, locks and occupancy. | Some messages are ~ 2 sec apart | Mixed | SSID, private IP, MAC, room/location and occupancy information. |
| sensor/# | Environmental sensors, energy/water data, GPS and health-related readings. | Messages appear ~ 1 sec to 5 sec apart | Mixed | GPS, device IDs, heart rate and SpO₂. |
| device/# | Device status, CCTV events, GPS, health data, relays, doors/gates and industrial telemetry. | Does not change frequently | Mixed | GPS, IP/MAC, device IDs, health and activity information. |
| test/# | Test/demo messages plus CCTV, GPS, sensor, energy and industrial-style examples. | Messages are more frequent | Mostly test | Some GPS, CCTV and technical information. |
| temperature/# | Temperature value and threshold value. | Does not change frequently | Mixed | No information |
| status/# | Online/offline status, firmware, private IP, Wi-Fi signal and gateway status. | Does not change frequently | Mixed | Private IP, firmware and device information. |
| data/# | PLC variables, motor parameters, Modbus registers, flow, power and SCADA/industrial data. | Some messages ~ 2 sec apart | Mixed | Industrial operational and technical information. |

One of the things that I observed is that the broker carried more than just the simple sensor values. There were certain topics which carried information regarding the devices and their surrounding environment. This included home/#, which carried information regarding lights, fans, doors and locks; sensor/#, which carried different types of sensor readings; and I even came across some information which included GPS and health information.

Status/# topic is useful for getting information about the devices in use. Some messages had such fields as firmware version, private IP and Wi-Fi signal strength. It could help one get to know what devices are used in the IoT environment. In the data/# topic, some messages appeared to be connected with industrial systems. The messages had information on PLC, Modbus, motors, processes and other things. It tells one that MQTT can be used not only in smart homes but in industrial IoT environments as well.

Moreover, some of the collected data was not resembling production data at all. There were topics whose content had such words as test, demo and hello, meaning that those topics were created in order to test the system. Consequently, it should not be claimed that every single device or message observed is actually part of an IoT deployment. The observations mainly show what kind of information can potentially be exposed through public MQTT topics.

One more important observation is that when I tried the # wildcard on HiveMQ, I did not receive the broad set of messages I expected. However, when I connected to test.mosquitto.org and subscribed to #, I received a very large amount of continuously changing data. The Mosquitto broker showed many different topics related to sensors, smart homes, solar systems, devices and industrial systems. I also observed topics such as SCADA, WTP and VOLT-AGE-PLC, showing industrial and monitoring-related data. This comparison showed me that the same MQTT wildcard can behave differently depending on how the broker is configured. Therefore, restricting or allowing broad subscriptions like # is mainly a matter of broker configuration and access control, not a difference in the MQTT protocol itself.

MQTT in Industrial and SCADA Environments

MQTT is not only used in smart homes and small IoT projects. It is also used in industrial environments to transfer information from sensors, machines and controllers to monitoring systems. During my analysis, the data/# topic contained PLC, Modbus, motor and SCADA-style information, so I also looked into the real world examples to know how MQTT is used in real industrial systems.

One example documented by Alias Robotics involved an MQTT broker used in a SCADA environment with Grafana. The broker did not require authentication, and researchers were able to subscribe to industrial sensor topics and inject false temperature and humidity readings. The changed values were displayed on the monitoring dashboard. This shows that an unsecured MQTT broker can affect the accuracy and trustworthiness of industrial monitoring data.

Another example was reported by OTORIO during research into InHand Networks' industrial cloud platform. They found MQTT broker configuration problems combined with other weaknesses, which could ultimately allow remote code execution on cloud-managed industrial routers. This shows that MQTT security problems can become more serious when combined with other weaknesses in an IoT system.

Trend Micro also found many publicly accessible MQTT brokers without authentication and observed real-world uses including industrial temperature monitoring, warehouse inventory and other IoT applications. This shows that exposed MQTT services are not limited to demonstrations or laboratory environments.

These examples show why the industrial data under data/# should be protected. If an industrial MQTT broker is left open, an attacker may be able to read sensitive operational information and, depending on the permissions configured, potentially interfere with the data being used by monitoring or control systems.

Why is it a security risk?

The main security problem I noticed is information exposure. If anyone can subscribe to useful MQTT topics, they may be able to see information about devices without being authorized to do so. Device IDs, IP addresses and firmware information can help identify devices. GPS data can reveal location, while occupancy, CCTV, door and lock information can reveal activity in a physical place. Industrial information can also reveal details about how a system is operating.

I also learned that simply blocking the # wildcard does not necessarily mean that the MQTT broker is secure. If specific and predictable topics such as home/#, sensor/# or device/# are still accessible, someone may still be able to collect useful information.